National Cheng Kung University

Department of Electrical Engineering

Introduction to VLSI CAD (Spring 2023)

Lab Session 4

Register Files, Manhattan Distance and LFSR

| Name | Student ID |  |
| --- | --- | --- |
| 陳慕丞 | E94096097 |  |
| Practical Sections | Points | Marks |
| Prob A | 25 |  |
| Prob B | 25 |  |
| Prob C | 25 |  |
| Report | 15 |  |
| File hierarchy, naming…etc. | 5 |  |
| Superlint | 5 |  |
| Notes: |  |  |

Due Date: 15:00, March 23, 2020 @ moodle

Deliverables

- All Verilog codes including testbenches for each problem should be uploaded.

NOTE: Please DO NOT paste source code in the report!

- Noted! TA will use commands in Appendix A to check your design in SoC Lab. If TA can not compile your code with the commands, you will not get full credit.

- If you upload a dead body which we can’t even compile, you will get NO credit!

- All Verilog file should get at least 85% SuperLint Coverage.

- All homework requirements should be uploaded in this file hierarchy or you will not get full credit.

NOTE: Please DO NOT upload waveforms!

Fig.1 File hierarchy for Homework submission

Prob A: Register File

- Based on the register file structure in LabA, please design a 64 x 32 register file by yourself.

- Port list

- You should follow the file name rules as follow.

- Register file

  - File name: register.v

  - Module name: register

- Register file testbench

  - File name: register_tb.v

- Show waveforms to explain that your register work correctly when read and write.

當reg_write=1，clock falling edge送入write_data時，下一個cycle的clock rising edge將write_data寫入REG_FILE[write_addr]，如上圖所示。

當reg_write=0，clock falling edge送入src1_addr和src2_add時，下一個cycle的clock rising edge將REG_FILE[src1_addr]輸出到src1，REG_FILE[src2_add]輸出到src2，如上圖所示。

- Show SuperLint coverage

Coverage:100%

Prob B: Finding Smallest Distance

Please design a circuit that will find the smallest distance between the local registers and the input data, based on the structure given in the LAB4 slide.

Port list

- Show waveforms to explain that your design works correctly.

Input: clk, rst, set_reg, color_val, data_val, color, data

Output: d, index

當set_reg和color_val為1時，將color存進color_reg [counter]中，並將counter加1；當data_val為1時，將1存進data_val_flag，將data存進data_reg，將counter的值存進counter_reg(記住存到第幾個color_reg)；當data_val或data_val_flag為1時，counter每個cycle減1(每個cycle用color_reg [counter]中的值比較)，clk_num每個cycle加1(負責數是否到第8個cycle)；運算中blue為|Bi-Bj|，green為|Gi-Gj|，red為|Ri-Rj|；若比較後距離等於或小於d中的值，則將距離的值存進d中，counter的值存進index中；最後當clk_num=7時，重設counter為counter_reg中的值，data_val_flag為0。

- Show SuperLint coverage

Coverage:100%

Prob C: LFSR

- Please design an 8-bit-LFSR, with the given feedback function in the LAB4 slide.

- Port list

- Feedback function

- Show waveforms to explain that your LFSR module works correctly.

當seed_val=1時，d被initial成seed的值7，如上圖所示，當seed_val=0時，d為{d’[6:0],((d’[7]~^d’[5])^(d’[4]~^d’[3]))} (上一個cycle的d值為d’)。

- Show the simulation result on the terminal.

- Show SuperLint coverage

Coverage:100%

- At last, please write the lesson you learned from Lab4

在Lab4中我學到如何實作sequential電路並應用在register,LFSR等電路中，並利用sequential電路做運算，也學到如何結合combinational和sequential電路實作比絕對值距離大小。

Appendix A : Commands we will use to check your homework