National Cheng Kung University

Department of Electrical Engineering

Introduction to VLSI CAD (Spring 2023)

Lab Session 5

Synthesis of Sequential Logic and Some Tips

| Name | Student ID |  |  |
| --- | --- | --- | --- |
| 陳慕丞 | E94096097 |  |  |
| Practical Sections: |  | Points | Marks |
| ProbA |  | 30 |  |
| ProbB |  | 35 |  |
| ProbC |  | 35 |  |
| Notes |  |  |  |

Due Date: 14:59, March 22, 2023 @ moodle

Deliverables

- All Verilog codes including testbenches for each problem should be uploaded.

NOTE: Please DO NOT include source code in the paper report!

- All homework requirements should follow the naming rule in this file hierarchy or you will not get the full credit.

NOTE: Please DO NOT upload waveforms!

- Important! TA will use the command in Appendix A to check your design under SoC Lab environment, if your code can not be recompiled by TA successfully  using the commands, you will not get the full credit.

- If you upload a dead body, which we cannot even compile, you will get NO credit!

- All Verilog file before synthesizing should get at least 95% Superlint Coverage.

- Lab5_Student_ID.tar (English alphabet of Student_ID should be capital.)

Lab5_studentID.tar

Lab5_studentID

studentID.docx

ProbA

det_seq.v

det_seq_tb.v

det_seq_syn.v

det_seq_syn.sdf

ProbB

mini_vending.v

mini_vending_tb.v

mini_vending_syn.v

mini_vending_syn.sdf

ProbC

pattern_gen.v

pattern_gen_syn.v

pattern_gen_syn.sdf

pattern_gen_tb.v

Fig.1 File hierarchy for Homework submission

Objectives:

To make you be familiar with some designs of sequential logic and Design Complier. You can follow this document to practice. Please show your best.

Note that you can extend the spacing if it is not enough for you to answer.

All labs should be synthesized, and clock period should not over 10 ns.

ProbA: Design a circuit “detecting pattern 11001”

- Design a pattern seq-detecting circuit that can be synthesized with moore machine. The following is det_seq module’s specification. (Do NOT add or delete I/O ports, but you can change their behavior.)

- Please describe your FSM in detail

Explanation about your FSM

IDLE會直接到FIRST 當d符合11001的input順序時，state順序為FIRST->SECOND->THIRD->FORTH->FITH->VALID，否則，當11001其中一個不符合時會出現不同的狀況，FIRST不符合時會回到FIRST，SECOND不符合時會回到FIRST，THIRD不符合時會回到THIRD，FORTH不符合時會回到SECOND，FITH不符合時會回到FIRST，到VALID時會拉起q並將num加1，VALID會有兩種情況，當input為1時會和前一個input形成11，因此會回到THIRD，否則當input為0時回到FIRST從頭開始。

- After synthesizing your design, you may have some information about the circuit. Please fill in the following form.

| Timing (slack) | Area (total cell area) | Power (total) |
| --- | --- | --- |
| 3.51 | 381.914994 |  |

- Please attach your design waveforms.

Your simulation result on the terminal.

RTL: Synthesys:

Your waveform :

RTL: Synthesys:

Explanation of your waveform :

Input:clk,rst,d Output:q,num 當rst時state為IDLE，next_state為FIRST，當next_state換狀態時，在下一個clk的posedge state就會跟著換狀態。當d符合11001且狀態順利走到VALID時q就會拉起且num加1。

Superlint Coverage

Coverage:100%

ProbB: Design a mini vending machine

- Design a mini vending machine with moore machine. The following is mini vending machine module’s specification. (Do NOT add or delete any I/O ports, but you can change their behavior.)

- Please describe your FSM in detail.

Explanation about your FSM

一開始為IDLE，然後直接進入MONEY，當en拉起時代表money輸入，因此進入BAVERAGE，否則回到MONEY，進入BAVERAGE時把 money_reg-beverage存進 money_reg，當en下降時代表baverage輸入，因此進入CHANGE，否則回到BAVERAGE，進入CHANGE時會拉升finish並將 money_reg輸出到change，之後再回到MONEY。

- After synthesizing your design, you may have some information about the circuit. Please fill in the following form.

| Timing (slack) | Area (total cell area) | Power (total) |
| --- | --- | --- |
| 1.03 | 675.565201 |  |

- Please attach your design waveforms.

Your simulation result on the terminal.

RTL: Synthesys:

Your waveform :

RTL: Synthesys:

Explanation of your waveform :

Input: clk, rst, en, money, beverage Output: change, finish 當rst時state為IDLE，next_state為MONEY，當next_state換狀態時，在下一個clk的posedge state就會跟著換狀態。當en拉起時state會到BAVERAGE並將money存到money_reg中，當en下降時state會到CHANGE，money_reg存進money_reg-beverage，finish拉起並將money_reg輸出到change，之後再回到MONEY。

Superlint Coverage

Coverage:100%

ProbC: Design a pattern generator

- Design a pattern generator which can create pattern 1100, 1101, 1110, 1111 and use mealy machine. The following is pattern generator specification.

- Please describe your FSM in detail.

Explanation about your FSM

一開始為IDLE，若en下降則回到IDLE，否則en拉升則進入FIRST(pattern輸出1)->SECOND(pattern輸出1)->THIRD(pattern輸出sel[1])->FORTH(pattern輸出sel[0])->ENDD，在ENND中，若en拉起則回到ENDD，否則en下降則進入LOW，在LOW中若en下降則回到LOW，否則en拉起則回到FIRST。另外，當在IDLE、ENDD、LOW時valid輸出0，其餘的輸出1。

- After synthesizing your design, you may have some information about the circuit. Please fill in the following form.

| Timing (slack) | Area (total cell area) | Power (total) |
| --- | --- | --- |
| 3.35 | 224.056798 |  |

- Please attach your design waveforms.

Your simulation result on the terminal.

RTL: Synthesys:

Your waveform :

RTL: Synthesys:

Explanation of your waveform :

Input: clk, rst, en, sel Output: pattern, valid 當rst時state為IDLE，next_state為IDLE，當next_state換狀態時，在下一個clk的posedge state就會跟著換狀態。在en拉起時state進入FIRST，當state在FIRST、SECOND時，pattern輸出1，當state在THIRD時，pattern輸出sel[1]，在FORTH時，pattern輸出sel[0]，當pattern輸出完畢時state進入ENDD，當en下降時state進入LOW，當en再次上升時state就再次進入FIRST。Valid只在state為FIRST、SECOND、THIRD、FORTH時拉升，其餘為下降。

Superlint Coverage

Coverage:100%

- At last, please write the lessons learned from this lab session, or some suggestions for this lab session. Thank you.

在此lab我學到了moore和mealy狀態機的差別和如何分別實作出來，也學到如何使用Verdi查看code的FSM圖。

Appendix A : Commands we will use to check your homework